AirTouch: A Low-Cost Surface-Less Touch Interaction Framework Using Sensor Fusion and Wireless Event Streaming

Pavan Karthikeya, Ayushmaan D. J. Neog, Jery Jayakumar, Yashwantha Rayalu G. V., Ranveer Kakati

School of Computer Science and Engineering (Artificial Intelligence), JAIN (Deemed-to-be University), Bengaluru, India

Abstract—This paper documents AirTouch, a Project Centric Learning (PCL) initiative focused on converting ordinary surfaces into interactive touch interfaces at minimal cost. The intended user outcome is straightforward: make classroom whiteboards, projected screens, and legacy displays behave like touch-enabled systems without replacing existing infrastructure. The repository contains a full software simulation stack, including geometric sensing simulation, noisy measurement handling, coordinate mapping, API transport, and cursor actuation on a host machine. Because physical hardware was not available in the lab during this phase, the current work is simulation-driven and should be interpreted as a pre-deployment validation stage rather than a finished embedded product. We present the system architecture, signal model, implementation details from every project artifact, and a reproducible Monte Carlo evaluation aligned with the implemented sensor assumptions. Under the evaluated setup, the prototype produced a valid-position rate of 99.18% and a median positional error of 0.4185 cm, with tail degradation under injected outliers. The work establishes technical feasibility, clarifies design trade-offs, and defines an actionable transition path toward hardware realization.

Index Terms—touchless interface, Time-of-Flight, ultrasonic simulation, sensor fusion, human-computer interaction, low-cost assistive computing

I. INTRODUCTION

Touch interaction has become a default expectation in education, productivity, and public information systems. The practical barrier is cost: most institutions cannot afford wholesale replacement of existing displays with dedicated smart boards. AirTouch explores a lower-cost direction—attach a compact sensing module, infer touch position from distance measurements, and convert that estimate into standard pointer events.

The project started from a two-sensor triangulation concept (fixed baseline, finger-distance inference, wireless click-event transmission). Over time, the software prototype evolved into a broader simulation stack that includes six virtual sensors, outlier modeling, smoothing, and receiver-side cursor stabilization. This paper consolidates both layers of the project: the original concept and the implemented simulation pipeline.

II. PROJECT CONTEXT, OBJECTIVES, AND CONSTRAINTS

AirTouch was developed as part of a multi-year PCL track. The core objective is to democratize touch interaction by reducing deployment cost to roughly ₹200–₹1000 per board-scale setup (hardware estimate for a future physical version). The intended social use case is highly practical: convert existing classroom whiteboards or projection areas into interactive surfaces.

During this project cycle, physical prototyping was constrained by hardware availability in the college lab. As a result, implementation effort focused on: (1) mathematical modeling, (2) end-to-end software simulation, (3) transport and input-event emulation, and (4) architectural readiness for later embedded integration.

III. RELATED WORK POSITIONING

The archived presentation and report documents in the repository consistently place AirTouch alongside prior touchless interaction research involving ToF ranging, depth sensing, mid-air feedback, and gesture recognition. The reviewed literature highlights four recurring pain points: limited precision, environmental sensitivity, range instability, and restricted gesture vocabulary. AirTouch directly addresses these through signal smoothing, configurable sensor orientation, and a modular transport pipeline that can host stronger recognition models later.

IV. SYSTEM OVERVIEW

A. Intended Physical Architecture

The design intent is a compact sensing bar mounted near the target surface. Early documents describe a two-sensor baseline model (approximately 5 cm sensor separation) with wireless communication via 2.4 GHz/Bluetooth and host-side click-event injection. Candidate sensor technologies considered include ToF, ultrasonic, laser, and IR modalities, with explicit cost–latency–accuracy trade-off analysis in project slides.

B. Implemented Software Architecture

The current repository implements a simulation-first architecture:

1) Sensing simulation and triangulation: `simulations/simulation.py` and `simulations/simulation.ipynb` model geometry, detection cones, noise/outliers, and optimization-based position recovery.

2) Event bridge: `simulations/python_bridge.py` forwards simulated position/click events to a local API at controlled update rates.

3) Wireless transport emulator: `simulations/api.js` receives coordinates, maps cm to pixels, injects packet effects, and broadcasts WebSocket mouse packets.

4) Host cursor receivers: `simulations/receiver.js` (Node + RobotJS) and `simulations/Receiver.java` (Swing visual receiver) consume packets and render/move pointer state.

5) Debug harness: `simulations/debug_test.py` sends deterministic movement patterns to validate API/receiver connectivity.

C. Key Observed Implementation Quirks

The codebase preserves multiple evolution stages. A few notable quirks are relevant for reproducibility: (i) both ToF and HC-SR04-style assumptions appear across files; (ii) source dimension constants differ between conceptual and implementation modules (e.g., 200×150 cm vs. 34.5×19.4 cm mapping assumptions); (iii) naming around interaction/mouse concepts is partially mixed (`interaction` vs `mouse`). These do not invalidate the architecture but should be normalized before production hardware integration.

V. MATHEMATICAL FORMULATION

For each sensor i at position s_i and finger position p, the measured distance follows

d_i = ||p − s_i||_2 + ε_i,

where ε_i captures Gaussian sensor noise and occasional outliers.

Given at least two valid observations, the simulation estimates p by minimizing squared residuals:

p* = argmin_p Σ_i (||p − s_i||_2 − d_i)^2, subject to screen-bound constraints.

The two-baseline closed-form (used in simplified reasoning) is

x = (d_A^2 − d_B^2 + W^2) / (2W),   y = sqrt(max(d_A^2 − x^2, 0)),

with W as baseline separation.

Finally, coordinate mapping to host display uses linear scaling:

x_px = round((x_cm / W_cm) · W_px),   y_px = round((y_cm / H_cm) · H_px).

VI. REPOSITORY-WIDE ARTIFACT AUDIT

All non–Base Papers artifacts were reviewed, including source code, notebooks, slides, reports, and image assets.

A. Root Files

• `AirTouch-Research.html`: historical design conversation export; captures original concept decisions and rationale.

• `final_paper.md`: initially empty draft placeholder.

• `Sem 5. Presentation for Lab.pptx`, `Year 1. 1st PPT - Team-3_PCL_Presentation.pptx`, `Year 1. 2nd PPT.pptx`, `Year 1. Final PPT - Final.pptx`, `Year_2._Final_PPT.pptx`: progressive project storytelling, literature references, methods, and requirement evolution.

• `Year 2. Final DOC.docx` and `Year 2. Final DOC - copy.docx`: earlier chapter-structured project report containing motivation, literature review, implementation plan, and architecture diagrams.

B. Algorithms and Simulation

• `algorithms/gesture_detection.ipynb`: empty notebook scaffold.

• `simulations/simulation.ipynb`: multi-cell evolution of the simulation environment (basic model to configurable sensor arrays and logging).

• `simulations/simulation.py`: integrated interactive simulation with dual array layout, cone overlap support, movement-angle estimation, data logging, and control widgets.

• `simulations/simulation-2.py`: alternate geometry simulation emphasizing angle-weighted estimation and animation loop behavior.

• `simulations/python_bridge.py`: asynchronous bridge from simulation events to API endpoint.

• `simulations/api.js`: HTTP + WebSocket transport, packet model, coordinate conversion, battery simulation, and packet-loss effects.

• `simulations/receiver.js`: real pointer control path with smoothing queue and coordinate clamping.

• `simulations/Receiver.java`: visualization receiver for debugging packet semantics.

• `simulations/debug_test.py`: scripted circular movement and periodic click generator for endpoint verification.

• `simulations/package.json` and `simulations/package-lock.json`: Node dependency graph (`express`, `ws`, `robotjs`).

C. Branding Assets

• `assets/AirTouch-Logo.png` (800×800), `assets/AirTouch-Logo-cropped.png` (806×382), `assets/AirTouch-Logo.psd` (800×800, RGBA) provide project identity resources for documentation and UI.

VII. EVALUATION METHODOLOGY

A reproducible Monte Carlo replay was conducted using the same geometry and noise assumptions reflected in `simulation.py`: screen width 35.0 cm, height 19.6875 cm, six directional sensors arranged as two top-corner arrays, 30° cone width, minimum/maximum sensing range 2–41 cm, Gaussian noise σ=0.3 cm, and outlier probability 2%.

Because the available environment lacked SciPy in the default terminal runtime, an analytic two-baseline estimator (left-array/right-array distance averaging + closed-form triangulation) was used for replay consistency. This mirrors the project’s baseline geometric logic while preserving the implemented noise model.

VIII. RESULTS AND DISCUSSION

A. Monte Carlo Summary (N = 6000 samples)

• Valid estimates: 5951 / 6000 (99.18%).

• Mean error: 0.7014 cm.

• Median error: 0.4185 cm.

• 90th percentile error: 1.1402 cm.

• 95th percentile error: 1.9564 cm.

• Maximum observed error: 14.0168 cm (tail event under injected outlier/noise conditions).

B. Sensor Activity Pattern

• Average active sensors per sample: 1.9918.

• Activity distribution: 49 samples with one active sensor, 5951 samples with two active sensors.

• Mean left-array activations: 0.9962; mean right-array activations: 0.9957.

C. Interpretation

Median-level behavior indicates stable interaction quality for typical conditions. The long-tail error is expected under deliberate outlier injection and should be handled in future hardware firmware through robust filtering, confidence gating, and temporal consistency checks before click commitment.

IX. COST, DEPLOYMENT, AND CLASSROOM IMPACT

The strongest practical contribution of AirTouch is economic: enable interactivity without replacing existing display infrastructure. In classroom contexts, retrofitting whiteboards/projected surfaces with a low-cost sensing module can drastically reduce deployment friction compared to full smart-board replacement. The same principle extends to legacy laptops and office displays, where touch-like interaction can be introduced as an add-on rather than a device upgrade.

X. LIMITATIONS, VALIDITY THREATS, AND ETHICAL POSITION

1) No physical hardware realization in this cycle; findings are simulation-backed.

2) Environmental effects (strong sunlight, reflective artifacts, occlusions) were modeled only partially.

3) Gesture vocabulary remains basic in current pipeline (hover/move/click emphasis).

4) Security, privacy, and system-hardening policies are architectural recommendations at this stage, not yet production-validated controls.

To maintain research integrity, claims in this manuscript are restricted to what is demonstrated in code and replayable simulation. Hardware-level performance claims are intentionally deferred to future work.

XI. CONCLUSION AND NEXT STEPS

AirTouch demonstrates a coherent and technically credible path to low-cost surface-less interaction. The current repository already contains a full software chain from sensing simulation to host-side pointer events, with configurable geometry, transport, and smoothing. Quantitative replay confirms strong median performance with identifiable tail-risk behavior under synthetic outliers.

Immediate next steps are: (i) procure and integrate physical sensors (ToF and/or ultrasonic), (ii) unify coordinate and calibration models across all modules, (iii) implement confidence-aware event gating, and (iv) benchmark real-world latency and error on classroom surfaces. With these steps, AirTouch can move from simulation maturity to deployable prototype status.

XII. SINGLE-COLUMN SUBMISSION READINESS NOTE

This document is intentionally prepared in single-column format and structured to be compatible with institutional publication workflows that request IEEE-style technical writing without mandatory two-column camera-ready constraints.

REFERENCES

[1] Gagana H. et al., “Innovation of Touchless Touchscreen Technology in Automotive User Interface,” Journal of Data Mining and Management, 2023.

[2] K. Yoshino et al., “Contactless Touch Interface Supporting Blind Touch Interaction by Aerial Tactile Stimulation,” 2023.

[3] M. E. Dominessy, “A Literature Review and Assessment of Touch Interactive Devices,” U.S. Army Human Engineering Laboratory, 1989.

[4] Kalaiselvi N. et al., “Touch Less Touch Screen,” International Journal of Recent Scientific Research, 2019.

[5] D. Pandav, “Haptic Technology,” IRJMETS, 2022.

[6] D. R. Schlegel et al., “AirTouch: Interacting with Computer Systems at a Distance,” 2011.

[7] N. Kumarasinghe, “AirTouch – Intelligent Virtual Multi-touch Screen,” 2017.

[8] D.-S. Lee, “Virtual Touch Sensor Using a Depth Camera,” 2019.

[9] A. Shtarbanov et al., “Soft Robotics and Programmable Materials for Human-Computer Interaction,” ACM DIS, 2023.

[10] C. E. Tejada, “AirTouch: 3D-Printed Touch-Sensitive Objects using Pneumatic Sensing,” CHI, 2020.

[11] H. Choi et al., “Integrated Pneumatic Sensing and Actuation for Haptic Devices,” IEEE, 2023.

[12] T. Yue et al., “Thermotion: Design and Fabrication of Thermofluidic Composites for Animation Effects on Object Surfaces,” CHI, 2023.

APPENDIX A: REPRODUCIBILITY SNAPSHOT

• OS context: Windows workspace.

• Repository root: c:/Users/jery/OneDrive/Documents/JAIN/PCL.

• Non-base artifacts audited: code (.py/.js/.java), notebooks (.ipynb), docs (.pptx/.docx/.html/.md), and assets (.png/.psd).

• Node stack in simulation folder: Express + WS + RobotJS.

• Evaluated display assumption in simulation: 15.6-inch class (≈35.0 cm × 19.6875 cm).

• Transport path: simulation event → HTTP `/position` → WebSocket broadcast → receiver smoothing/cursor actuation.